### MICROPROCESSOR TRAINER KIT

### ALS-SDA-85ME

| CPU | 8085 Operating @ 3.072 MHz |
| --- | --- |
| MEMORY | 64KB MAX (32KB EPROM and 32KB RAM) Battery Backup option for RAM |
| NOTE | The system is supplied with 16KB EPROM and 8KB RAM |
| I/O PARALLEL | 48 I/O lines using two 8255 |
| I/O SERIAL | 0ne RS232 compatible interface |
| TIMER | Three 16 bit counter /timer using 8253 |
| KEYBOARD | Consisting of 28 numbers of computer grade keys |
| DISPLAY | Six numbers of seven segment displays |
| BUS SIGNALS | All Address, Data and Control signals are terminated in 50 pin berg stick for user expansion |
| PIC | Optional facility for 8259. All the 8 interrupts are terminated in berg stick |
| MONITOR SOFTWARE | 16KB of powerful user friendly monitor software with keyboard and serial modes |

### ALS-SDA-85MEL

| CPU | 8085 Operating @ 3.072 MHz |
| --- | --- |
| MEMORY | 64KB MAX (32KB EPROM and 32KB RAM) Battery Backup option for RAM |
| NOTE | The system is supplied with 16KB EPROM and 8KB RAM |
| I/O PARALLEL | 48 I/O lines using two 8255 |
| I/O SERIAL | 0ne RS232 compatible interface |
| TIMER | Three 16 bit counter /timer using 8253 |
| KEYBOARD | External PC-AT compatible keyboard |
| DISPLAY | 16 characters 2 line JUMBO LCD display |
| BUS SIGNALS | All Address, Data and Control signals are terminated in 50 pin berg stick for user expansion |
| PIC | Optional facility for 8259. All the 8 interrupts are terminated in berg stick |
| MONITOR SOFTWARE | 16KB of system monitor, which allows the user to enter, verify, debug or execute the program either from the on-board PC keyboard and display or through serial mode. On-line assembler using PC keyboard and LCD display |

### ALS-SDA-85LC

| CPU | 8085 Operating @ 3.072 MHz |
| --- | --- |
| MEMORY | EPROM: One JEDEC compatible 28-pin socket provide up to 32K bytes. RAM :One JEDEC compatible 28-pin socket provide up to 32K bytes of CMOS static RAM. Provision to battery back the entire RAM (Optional). |
| I/O PARALLEL | 24 I/ O lines using one 8255 terminated in 26 pin berg headers |
| I/O SERIAL | One RS-232C compatible interface, using USART-8251A with programmable baud rates through 8253 programmable timer. These lines are terminated in a 9-pin D type female connector. |
| TIMER | Three 16-bit counter/timer using 8253 programmable timer. Counter 1 is used for serial I/O baud rate generation. |
| INTERRUPT | RST7.5 is used to provide keyboard VECT-INTR function. TRAP is used for single instruction step function RST5.5 is used for keyboard operation. RST6.5 and INTR are available to user. RST 7.5 and TRAP are available to the user if VECT-INTR & SINGLE STEP functions are disabled. |
| KEYBOARD | Consists of 28 numbers of keys with tops. |
| DISPLAY | Six numbers of bright seven-segment displays for address and data. |
| BUS SIGNALS | All Address, Data and Control signals are terminated in 50 pin berg stick for user expansion. All signals are marked with suitable legends. |
| PIC | Optional facility for 8259. All the 8 interrupts are terminated in berg stick |
| MONITOR SOFTWARE | 16K bytes of user friendly monitor software that allows programs entry, verification, debugging and execution from the on-board keyboard / serial mode. Line assembler/ disassembler, file upload / download options are through serial mode. |
| POWER SUPPLY | Power adapter socket provided. |

### ALS-SDA-85HB

| CPU | 8085 Operating @ 3.072 MHz |
| --- | --- |
| MEMORY | 64KB MAX (32KB EPROM and 32KB RAM) Battery Backup option for RAM |
| NOTE | The system is supplied with 16KB EPROM and 8KB RAM |
| I/O PARALLEL | 24 I/O lines using 8255 |
| I/O SERIAL | Optional 0ne RS232 compatible interface |
| TIMER | Three 16 bit counter /timer using 8253 |
| KEYBOARD | Consisting of 28 numbers of computer grade keys |
| DISPLAY | Six numbers of seven segment displays |
| BUS SIGNALS | All Address, Data and Control signals are terminated in 50 pin berg stick for user expansion |
| MONITOR SOFTWARE | 16KB of powerful user friendly monitor software with keyboard and serial modes |

### ALS-SDA-86ME

| CPU | 8086 Operating @ 5MHz in MAX mode with provision for 8087 Co-processor |
| --- | --- |
| MEMORY | Maximum of 128KB on board EPROM. Maximum 64KB of on board RAM. Battery Backup option for RAM |
| I/O PARALLEL | 48 I/O lines using two 8255 |
| I/O SERIAL | 0ne RS232 compatible interface |
| TIMER | Three 16 bit counter /timer using 8253 |
| KEYBOARD | Consisting of 28 numbers of computer grade keys |
| DISPLAY | Eight numbers of seven segment displays |
| BUS SIGNALS | All Address, Data and Control signals are terminated in 50 pin berg stick for user expansion |
| PIC | Programmable Interrupt controller using 8259 |
| MONITOR SOFTWARE | 16KB of powerful user friendly monitor software with keyboard and serial modes |

### ALS-SDA-86MEL

| CPU | 8086 Operating @ 5 MHz in MAX MODE with provision for 8087 co-processor |
| --- | --- |
| MEMORY | Maximum of 128 kb on board EPROM MAXIMUM OF 64 KB ON BOARD RAM. Battery Backup option for RAM |
| NOTE | The system is supplied with 64KB EPROM and 64 KB RAMand 8KB RAM |
| I/O PARALLEL | 48 I/O lines using two 8255 |
| I/O SERIAL | 0ne RS232 compatible interface |
| TIMER | Three 16 bit counter /timer using 8253 |
| KEYBOARD | External PC-AT compatible keyboard |
| DISPLAY | 16 characters 2 line JUMBO LCD display |
| BUS SIGNALS | All Address, Data and Control signals are terminated in 50 pin berg stick for user expansion |
| PIC | PROGRAMMABLE INTERRUPT CONTROLLER USING 8259 |
| MONITOR SOFTWARE | 64KB of system monitor, which allows the user to enter, verify, debug or execute the program either from the on-board PC keyboard and display or through serial mode. On-line assembler using PC keyboard and LCD display |